Laser safety

AMR ELDIB

Laser safety Topics

Lecture1-Introduction

Lecture 2-Fundamentals of lasers

Lecture 3-Laser operation and applications

Lecture 4-Hazards laser and non beam

Lecture 5-Bioeffects and Accidents

Midterm exam

Lecture 6-Laser standards

Lecture 7-ANSIZ136.1 standards

Lecture 8-Control measures

Lecture 9-Laser safety rules

Lecture 10- MPE,OD&NHZ examples

4- LASER HAZARDS

1- Laser beam hazards                                                             Non beam radiation hazards

Skin                Eye

Chapter

4

LASER HAZARDS

Laser beam hazards                                                             Non beam radiation hazards

Skin                Eye

1-Skin

200-315 nm actinic UV range from erythema to blistering , possible carcinogenic effects 315nm – 1mm  Thermal effects  range from mild reddening (erythema) to blistering, charring                             depends upon exposure time , exposure dose, rate and conduction of heat  away from site  of absorption

2- Eye

UV-A affects the lens   very high dose may produce opacity                                                               UV-B and UV-C affects   the cornea similar to the effects produced  by arc welders .    Acute inflammation of conjunctiva as well.

| 315-400nm | UV-A |
| --- | --- |
| 280 -315 nm | UV-B |
| 180-280nm | UV-C |

IR-B and IR-C  (far IR) affects the cornea and lens ,heat effect on ornea and lens .

| IR- A (NEAR-IR0 | 740-1400 nm |
| --- | --- |
| IR-B | 1400-3000 nm |
| IR-C (FAR- IR) | More than 3000 nm |

VISIBLE and NEAR-IR                                                                                                                         Passes through  the transparent media of the eye ( cornea ,lens, & vitreous) and affects the Retina  effects may be temporary or permanent ,lesion ranging from edema to charring with hemorrhage and tissue reaction around the lesion. With very high radiant energy tissue photodisruption of the physical structure of the eye can happen,

Non beam radiation hazards

Electric shock and electrocution

Air contamination ( laser gas or tissue vaporized particles)

Collateral and plasma generation

X ray radiation generation

UV & Visible plume radiation

Radiofrequencies (RF)

Fire hazards

Explosion hazards

Compressed gases

Laser dyes and cryogenic materials

Mechanical   accidents

Noise levels

Waste disposal

Visibility issues / confined space

Human factors /Ergonomics

Notes