ISSR

Dept. of Computer & Information Sciences

Dr. Abd El-Aziz Ahmed

Introduction to Computer Science

Spring term 2015

Sheet 3

- What is the total number of memory cells that CPU can access using 20-address line?  If the capacity of each memory cell is 16 bits, what is the total memory capacity in B?

- A hard disk has two storing sides, with 10 tracks per each side. Each track contains 5 sectors, and each sector contains 512 bytes. If the hard spins at the rate of 300 rpm and the computer machine can execute 60 instructions in a microsecond.

Find the following:

(a) The total storage capacity of the hard disk.

(b) The average access time if the seek time is 8 millisecond.

(c) The number of instructions that can be executed in the time between two consecutive

bytes passing the read/write head.

- Use the instructions set table of the hypothetical CPU to translate the                         following machine language program:

step1     146A

step2     136C

step3     5034

step4     306E

step5     C000

- Write the above program in symbolic form.

- Write each instruction of the above program in a bit pattern.

- LOAD R0, M[A6]

LOAD R1, E7

AND R2, R0, R1

LOAD R1, 42

OR R3, R2, R1

LOAD R1, 81

XOR R0, R3, R1

STORE R0, A6

HALT

Trace the execution of the following machine language program and write the content of

memory cell A6 at the end of the execution.

- Show the execution of the following program:

- It is required to isolate the most significant bits of a seven-bit string. What mask must be used with what operation?

- It is required to complement the middle three bits of a seven-bit string while leaving the other four bits undisturbed. What mask must be used with what operation?

- What logical operation together with what mask can be used to change ASCII codes of A to a, and vise-verse?

- Which of the following is not contained in a CPU?

A. Instruction register		B. Program counter  C. General-purpose register	D. Memory cell

- Which of the following instructions changes the contents of a memory cell?

A.  10AB	B.  20AB	C.  30AB	D.  40AB

- Which of the following instructions places 00000000 in register A?

A.  1A00	B.  2A00	C.  3A00	D.  200A

- Which of the following instructions places 00000000 in register 5?

A.  25FF		B.  9555		C.  15FF		D.  8555

- Which of the following instructions will not change the contents of register 5?

A.  1508		B.  2508		C.  A503	D.  A508

- Which of the following instructions is equivalent to requesting that register A be rotated to the left by three bits?

A.  AA05	B.  AA03	C.  AA08	D.  AA01

- Which of the following instructions changes the contents of register 7?

A.  4077		B.  4075		C.  4057		D.  37BB

- In which of the following locations is information most readily available for manipulation by the CPU?

A. Cache memory		B.  Main memory

C. Mass storage		D. General-purpose registers

- Which of the following instructions does not fall in the category of arithmetic/logic instructions?

A.  ROTATE	         B.  ADD		C.  OR		D.  JUMP

- Which of the following instructions falls in the category of data transfer instructions?

A.  LOAD	B.  AND		C.  ROTATE	D.  JUMP

- Which of the following is not a component of a machine instruction?

A.  Op-code	      B.  Port		C.  Operand

- Which of the following is not an activity performed entirely within a CPU?

A.  Fetch instructions		B.  Perform Boolean operations

C.  Perform arithmetic operations	D.  Move data between registers

- What mask in register F would cause the instruction 8AAF  to put a 0 in the most significant bit of register A without disturbing the other bits?

A.  11111110	B.  00000001	C.  10000000	D.  011111111

- What mask in register F would cause the instruction 7AAF to put a 1 in the most significant bit of register A without disturbing the other bits?

A.  11111110	B.  00000001	C.  10000000	D.  011111111

- Which of the following instructions will not produce the same result as the other two?

A.  A502	B.  A506	C.  A50A

- Which of the following instructions will not produce the same result as the other two?

A.  9555		B.  2500		C.  1500

- If register A contained the pattern 00000000, which of the following instructions could alter the contents of register 0?

A. 700A		B.  800A	C.  900A

- Which of the following instructions is essentially an unconditional jump?

A.  B033	B.  B133		C.  B233		D.  B333

- Which step of the machine cycle examines the op-code of the next instruction to be executed?

A.  Load		B.  Fetch	C.  Decode	D.  Execute

- If register 0 contains the pattern 01101001 before executing the instruction A003 , what bit pattern will be in register 0 after the instruction is executed?

- If registers 5 and 6 contain the bit patterns 5A and 58 respectively, what bit pattern will be in register 4 after executing the instruction 5456?

- If registers 5 and 6 contain the bit patterns 5A and 58 respectively, what bit pattern will be in register 4 after executing the instruction 6456? Assume a floating-point format in which the most significant bit is the sign bit, the next three bits represent the exponent field in excess notation, and the last four bits represent the mantissa.)

- Write the answer to each of the following logic problems.

10101010		   10101010		    10101010

AND 11110000		OR 11110000		XOR 11110000

- Suppose registers E and F contained AA and CC, respectively. What bit pattern would be in register D after executing each of the following instructions?

A.  7DEF	    B.  8DEF	    C.  9DEF

- If registers 0, 1, and 2 contain the patterns A5, A5, and B7, respectively, which of the following instructions will result in a jump to location AA?

A.  B0AA	B.  B1AA	C.  B2AA

- If registers 0 and 1 contain the patterns B5 and F0, respectively, what will be in register 1 after executing each of the following instructions?

A.  A102	B.  4001       C.  4010

- Suppose the instruction B1A5 is stored in main memory at addresses E0 and E1. Moreover, suppose registers 0 and 1 both contain the pattern FF. What value will be in the CPU’s program counter immediately after executing the instruction?

- Suppose the instruction B1A5 is stored in main memory at addresses E0 and E1. Moreover, suppose registers 0 and 1 contain the patterns FF and 75, respectively. What value will be in the CPU’s program counter immediately after executing the instruction?

- Encode each of the following commands in terms of the machine language described in the language description table.

- LOAD register 7 with the value A5.

B. LOAD register 7 with the contents of the memory cell at address A5.

C. ADD the contents of registers 5 and 6 as thought they were values in two’s

complement notation and leave the result in register 4.

D. OR the contents of registers 5 and 6, leaving the result in register 4.

- Encode each of the following commands in terms of the machine language described in the language description table.

A. __________ 	ROTATE the contents of  register 7 to the right 5 bit positions.

B. __________ 	JUMP to the instruction at address B2 if the content of register 2 equals that of

register 0.

C. __________ 	ADD the contents of registers 5 and 6 as thought they were values in floating-

point notation and leave the result in register 4.

D. __________	AND the contents of registers 5 and 6, leaving the result in register 4.

- Decode each of the following instructions that were encoded using the language description table.

A.  4034		B.  8023			C.  B288

D.  2345

- Decode each of the following instructions that were encoded using the language description table.

A.  A004		B.  1234			C.  5678

D.  C000

- The following table shows a portion of a machine's memory containing a program written in the language described in the language description table. Answer the questions below assuming that the machine is started with its program counter containing 00.

address   content

00     21

01     0B

02     14

03     04

04     C0

05     00

A.  What bit pattern will be in register 4 when the machine halts?

B.  What bit pattern will be in register 1 when the machine halts?

- The following table shows a portion of a machine's memory containing a program written in the language described in the language description table. Answer the questions below assuming that the machine is started with its program counter containing 00.

address   content		address   content

00    10			  07    00

01    02			  08    C0

02    24			  09    00

03    04			  0A    C0

04    B4			  0B    00

05    0A			  0C    C0

06    C0			  0D    00

A.  What bit pattern will be in register 0 when the machine halts?

B.  What bit pattern will be in register 4 when the machine halts?

C.  What bit pattern will be in the program counter when the machine halts?

- The following table shows a portion of a machine's memory containing a program written in the language described in the language description table. Answer the questions below assuming that the machine is started with its program counter containing 00.

address   content		address   content

00    25			  07    00

01    03			  08    C0

02    20			  09    00

03    F9			  0A    C0

04    53			  0B    00

05    05			  0C    C0

06    33			  0D    00

A.  What bit pattern will be in register 5 when the machine halts?

B.  What bit pattern will be in register 0 when the machine halts?

C.  What bit pattern will be in register 3 when the machine halts?

D.  What bit pattern will be at memory location 00 when the machine halts?

- The following table shows a portion of a machine's memory containing a program written in the language described in the language description table. Answer the questions below assuming that the machine is started with its program counter containing 00.

address   content		address   content

00    25			  07    00

01    03			  08    34

02    A5			  09    04

03    02			  0A    B0

04    35			  0B    03

05    03			  0C    C0

06    24			  0D    00

A.  What bit pattern will be in register 5 when the machine halts?

B.  What bit pattern will be in the program counter when the machine halts?

C.  What bit pattern will be at memory location 04 when the machine halts?

- Below is a short routine written in the machine language described in the language description table and stored in a machine's memory beginning at address 50. What must be in the memory cell at address 40 to avoid an unending loop?

Address    Instruction

50	        2001

52	        1340

54	        8330

56	        B352

58	         ...

- The following table shows a portion of a machine's memory containing a program written in the language described in the language description table. Answer the questions below assuming that the machine is started with its program counter containing 00.

address   content		 address   content

00    B0			  07    C0

01    03			  08    00

02    25			  09    23

03    B0			  0A    B0

04    0C			  0B    03

05    C0			  0C    B0

06    00			  0D    07

A. How many instructions will be executed before the machine halts?

B. What bit pattern will be in the program counter when the machine halts?

- The following table shows a portion of a machine's memory containing a program written in the language described in the language description table. Answer the questions below assuming that the machine is started with its program counter containing 00.

address   content	             address   content

00    20			  07    12

01    02			  08    B2

02    21			  09    0C

03    01			  0A    B0

04    22			  0B    06

05    01			  0C    C0

06    52			  0D    00

A.  What bit pattern will be in register 2 when the machine halts?

B.  How many times will the instruction at address 06 be executed before the machine halts?

- What is used to differentiate memory used for instructions and memory used for data?